# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Waste collection vehicle	p 1 /15

Publication reference: BSB00027-PP5-SUP-01

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

1 – GENERAL REQUIREMENTS

1.1 	General requirements applying to all of the items:

- All the equipment shall be provided complete with the necessary accessories and/or parts such as to ensure that the unit is capable of operating to the required technical and quality specifications. All specifications details listed is the minimum requirements. Any improvements on the specifications or additional features offered should be clearly identified in the Tenderer’s offer.

- The Tenderer shall confirm that the tendered equipment complies with applicable EU standards and certifications. Standards and codes including addenda shall be valid at the date of submitting the tender.

- The supplier will be responsible for advising of any Health & Safety risks associated with equipment provided and of suitable protective measures.

- The Tenderer shall clearly state in the attached detailed Technical Specifications the proposed model, trade mark, manufacturers and country of origin.

- All equipment must be supplied with full technical documentation comprising, at the minimum, operation manual, and service manual. This documentation must be supplied, at the time of delivery of the equipment, if applicable in the Turkish language, unless stated otherwise. It is the sole responsibility of the Tenderer to ensure that all supplies are certified as required. In addition to the requirements stipulated elsewhere in the tender document, the Tenderer shall with his tender provide brochures, leaflets and other documentation specifying the tendered supply in an unambiguous manner.

- Equipment which allows upgrading of capacities shall be provided in such a way that upgrades can be performed by installing additional capacity without discarding the already installed capacities.

- All equipment shall be delivered DDP (Delivered Duty Paid). The location for delivery for each item is will be following address;

Uzunkopru Municipality

Cumhuriyet Mah. 19 Mayıs Cad No 160

22200 Uzunkopru / Edirne, TÜRKİYE

1.2 	Putting into Operation of equipment

- Delivery and put into operation by the Contractor of the equipment is required.

- The Contractor shall submit with his tender a description of his proposed activities and activity schedule at the location of delivery for put into operation and testing of the equipment according to Article 25 of the Special Conditions of Contract.

- The Contractor shall perform on site installation of the supplied goods including unloading at the above-mentioned address.

- Installation and put into operation shall be conducted by properly trained and authorized personnel of the Contractor.

- The Contractor shall make available to himself all tools, materials and equipment required for in-house transport, assembling and putting in operation.

- The Contractor shall perform starting-up and commissioning of the equipment, furnishing of all required materials such as consumables needed for testing and initial operation of the supplied waste collection vehicle.

- The Tenderer shall, with his Tender provide a description of how maintenance and support will be provided during the required period following provisional acceptance as well as beyond this period, including postal address, telephone and facsimile number, e-mail address etc. of the service organisation. Maintenance and support shall be conducted by properly qualified technicians.

- The Tenderer shall also provide a confirmation of acceptance issued by the appointed maintenance service provider.

1.3	Training

- The contractor shall provide minimum 1 (one) day training session to max 15 (fifteen) staff of the Municipality of Uzunkopru. These staff will be trained about general operating rules for using the solid waste collection vehicle. Training will be carried out at a suitable place belonging to Municipality of Uzunkopru.

- The language of the training and the training documents will be in Turkish; if not, translation will be provided and paid by the Contractor.

- The contractor will be responsible to provide all the necessary service, technical personnel, product, equipment, documentation and training programs about general operating rules for the vehicle.

1.4	Basic Technical Specification (minimum requirements)

- Tenderers are to offer standard production models matching or exceeding the specifications stated in the outline specifications – see detailed Technical Specifications. The tenderer shall ensure that the functions and features of the equipment meet the listed minimum conditions and should submit equipment brochures and catalogues showing the specifications of the proposed solid waste collection vehicle with 8+1 m³ Capacity Hydraulic Compactor Garbage Body.

- All specifications details for each item are the minimum requirements. Any improvements on the specifications or additional features offered should be clearly identified in the Tenderer’s offer.

- Any goods which provide performance at least substantially equivalent to those specified in the Technical Specifications will be evaluated.

- Wherever reference is made in the Technical Specifications to specific standards and codes to be met by the goods and materials to be furnished or tested, the provisions of the latest current edition or revision of the relevant standards in effect, shall apply, unless otherwise stated in the Contract. Where such standards and codes are national or relate to a particular region, other authoritative standards that ensure substantial equivalence to the standards or codes specified will be evaluated.

- It should be noted that whenever a specific name of a product is mentioned in the Technical Specifications, a sufficiently precise and fully intelligible description is not possible, and it has to be understood as that product or its equivalent. All the equipment shall be provided complete with the necessary accessories and/or parts such as to ensure that the unit is capable of operating to the required technical and quality specifications.

1.5	Contractor’s Technical Offer

- All technical documentation supplied with the tender must match the Tenderer’s written specifications.

1.6	Technical Documents to be provided during the time of tendering and provicional acceptance

Each set of equipment shall be provided with an original set of technical manuals for all parts of the vehicle, including all accessories, as applicable. The manuals shall be supplied in Turkish and/or English language. If manuals are not in Turkish, a “Quick Guide” in Turkish shall be supplied together with the manual as applicable.

  - The following documents are requested during the tendering related to the manufacturing of the proposed garbage truck and equipment:

  - TSE Service Competence Certificate

  - TS EN 1501-1: Certificate of Conformity for Hydraulic Compactor Garbage Body

  - Industry Registration Certificate issued in the name of the Tenderer by the Ministry of Industry and Trade

  - Manufacturing Competence Certificate issued in the name of the tenderer by the professional chamber to which the tenderer is registered

- TS EN ISO 9001:2008 Quality Certificate

- Certificate of compliance with CE norms, as applicable;

The following documents are requested during the provisional acceptance related to the manufacturing of the proposed garbage truck and equipment:

- Guarantee document of manufacturer;

- Warranty document of the manufacturer and/or the supplier, as applicable;

- The tenderer should provide copy of quality assurance certificate issued by firm accredited by ISO 9001:2008 or an equivalent European quality mark, for the supplies offered, as applicable.

1.7	Visibility

The vehicle shall have a solidly fixed and durable label, size 75 mm x 35 mm, with the standard Inttereg logo according to the rules of the programme Interreg NEXT Black Sea Basin 2021-2027 programming period. The label should include all obligatory information described in the visualization manual of the programme. https://blacksea-cbc.net/interreg-next-bsb-2021-2027/project-toolkit/communication-and-visibility

Waste Collection Vehicle with 8+1 m³ Capacity Hydraulic Compactor Garbage Body for collection of recyclables solid waste within the territory of Uzunkopru municipality, Edirne Province, Türkiye – 1 unit.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | HYDRAULIC COMPACTOR GARBAGE BODY -1 |  |  |  |
|  | GENERAL DESCRIPTION a) The body volume of the garbage container shall be at least 8 m³. b) The rear garbage collection compartment shall have a capacity of at least 1 m³. c) The total container volume shall be at least 8+1 m³. d) The container weight shall be approximately 4000 kg. e) Garbage Container: The body/container shall consist of the main body, discharge shovel, rear compartment, compaction shovel inside the rear compartment, container lifting mechanism/system, hydraulic, electrical, and pneumatic components, and control systems. |  |  |  |
|  | BODY/CONTAINER |  |  |  |
|  | - Body Type: The body is of an oval type with an aesthetically pleasing appearance, featuring outwardly curved surfaces joined with lower press-formed profiles. - Body Sheet Thickness (Standard): - Sidewalls, base, and roof: 4 mm, St 52 (S355JR). - Lower Chassis: Manufactured from 5 mm St 52 steel. - Wastewater Tank: A wastewater tank with an approximate capacity of 70 liters shall be located beneath the container. |  |  |  |
|  | 3- DISCHARGE SHOVEL a) The front sheet/wall of the discharge shovel that comes into contact with the waste is manufactured from St 52 (S355JR) material. b) Polyamide plates will be used for bearing, with U-bent sheets or profiles. c) When the container is empty, the discharge shovel will be positioned at the very rear and will move forward as loading progresses. d) A double-acting telescopic cylinder will be used for discharge. e) Depending on the type of waste, the compression ratio will range between 1/3 and 1/6. |  |  |  |
|  | 4- REAR DOOR (REAR COMPARTMENT) a) The garbage collection compartment shall be entirely manufactured from high-strength steel sheets. - Rear compartment steel: 5 mm - Side front wall: 4 mm - Side rear wall: 3 mm - Material: St 52 steel b) A mechanically operated safety latch, ensuring the safety of both sides in a single motion, shall be provided to secure the rear door during maintenance and cleaning when the door is open. c) The rear compartment shall have a garbage collection capacity of at least 1 m³. d) The opening and closing of the rear door shall feature an automatic locking system. e) A safety locking mechanism shall be included to prevent the sudden dropping of the rear door due to issues such as hydraulic hose failures during garbage unloading. f) To prevent water leakage between the container and the rear door, an acid-resistant rubber gasket (sealing rubber) shall be used. |  |  |  |
|  | g) Steps and handrails (grip bars) shall be provided for operators/workers at the rear of the garbage truck. |  |  |  |
|  | 5- COMPACTION SHOVEL & SYSTEM a) The compaction shovel (bucket) sheets shall be manufactured from St 37 (S255JR) or equivalent materials. b) The slide shafts shall be mounted on a guide track using acid-resistant and wear-resistant polyamide pads. c) The slide cylinders shall be located outside the rear compartment and concealed with side covers. d) The movements of the slide and the compaction shovel shall be powered by double-acting cylinders. |  |  |  |
|  | 6- HYDRAULIC SYSTEM a) The PTO (Power Take-Off) gearbox connection to the transmission shall be air-controlled and operable from the driver’s cabin. b) All hydraulic cylinders shall be manufactured from honed (H8) steel-drawn tubes, with the cylinder rods ground and coated with hard chrome for corrosion resistance. c) Hydraulic pressure hoses shall be double steel-braided, of appropriate diameter, and conform to SAE 100R-1, R2, R4, and R9 standards. d) The hydraulic oil tank shall have a capacity of 100 liters and be equipped with a return filter, a µ60-grade tank cap, and an oil level indicator. e) The hydraulic system operating pressure shall be adjustable between 80 and 180 kg/cm², depending on operational requirements. f) The hydraulic pump shall be selected according to the vehicle's engine speed and PTO specifications. |  |  |  |
|  | 7- OPERATION a) Garbage in the container shall be unloaded into the rear compartment using a container lifting mechanism operated by two double-acting cylinders. b) The compaction shovel and slide cylinders shall operate in single or continuous cycles with the press of a button. These movements shall be controlled electronically. Control buttons and mechanical levers shall be located on the right side of the vehicle’s rear door (rear compartment). A stop button shall be provided on the left side of the rear compartment. c) The unloading process shall occur by opening the rear door and moving the unloading shovel backward. Control valves shall be located at the front left side of the container. d) A driver alert bell button and a stop button shall be installed on the left side of the rear compartment. e) To discharge any remaining garbage in the rear compartment during unloading, a start button shall be installed on the front left side of the container. While the garbage container operates, the vehicle's engine speed shall be maintainable at a fixed rate – the vehicle must feature a speed stabilization function. |  |  |  |
|  | 8- CONTAINER LIFTING MECHANISM A container lifting system, compliant with DIN standards, shall be installed on the rear door to allow the emptying of garbage containers with capacities of 0.8 m³ (770–800 liters) and 1.1 m³. (The lifting arm for 0.4 m³ containers is optional.) |  |  |  |
|  | 9- PAINTING a) The garbage container shall undergo a sandblasting process prior to painting. b) After being cleaned with a rust-preventive treatment, the container shall be primed and then painted with one coat of the desired color (specified by the RAL code). |  |  |  |
|  | 10- USER MANUAL A user manual containing all electrical and hydraulic schematics, spare parts lists, and operation and maintenance instructions shall be provided with the vehicle at the time of delivery. |  |  |  |
|  | 11- WARRANTY a) The garbage containers shall be covered under warranty for a period of two (2) years against steel construction defects, excluding damages caused by misuse or lack of regular periodic maintenance. The hydraulic and electrical components shall be covered for one (1) year. b) The garbage containers shall comply with the national traffic laws and regulations. |  |  |  |
| 2 | VEHICLE TECHNICAL SPECIFICATION -1 |  |  |  |
|  | DIMENSIONS - Length (mm): Minimum 6885 - Width (mm): Minimum 2300 - Height (mm): Minimum 2335 - Wheelbase (mm): Minimum 3800 |  |  |  |
|  | WEIGHT - Front Axle Technical Capacity (kg): Minimum 4000 - Rear Axle Technical Capacity (kg): Minimum 7600 - Carrying Capacity (kg): Minimum 7091 |  |  |  |
|  | ENGINE SPECIFICATIONS - Engine Type: Turbo Diesel Euro 6 - Displacement: Minimum 3800 cc - Number of Cylinders: 4 - Maximum Net Power: Minimum 122 kW @ 2600 rpm - Maximum Net Torque: Minimum 584 Nm @ 1500-1800 rpm - Transmission: Manual, 6 forward gears, 1 reverse |  |  |  |
|  | BRAKES - Service Brake: Fully dry air, EBS 3, ESC - Front Brake: Disc - Rear Brake: Disc - Park Brake: Pneumatic emergency spring-loaded, effective on rear wheels |  |  |  |
|  | STEERING - Steering: Hydraulic |  |  |  |
|  | ELECTRICAL EQUIPMENT - Batteries: Minimum 2 x 12 Volt, 105 Ah - Air Conditioning: The vehicle will be equipped with an air conditioning system |  |  |  |
|  | FUEL CAPACITY - Fuel Tank Capacity (L): Minimum 100 |  |  |  |